山岳组装教程
Assembly Tutorial for Mountain 
一：美纹纸贴入底壳凹槽处（随包装赠送）。防止任何情况的PCB短路情况。
Step 1: Use the yellow masking tape provided to line up along the groove of the bottom case. This prevents the PCB from getting short circuited. 
[image: image1.png]K416 ~

H 'n-rhnlrm llalﬂ ;
oo




二：将亚克力挡板和屏幕的保护膜撕掉。

Step 2.1: Remove the protective film from the clear acrylic piece.

[image: image2.png]



Step 2.2: Remove the protective film from the OLED Screen.

[image: image3.png]



三：将亚克力挡板与屏幕先后放入上盖中，拧紧后面的金属挡板。

Step 3: Place the acrylic piece and the OLED screen on top case OLED cutout and use the metal block to seal and tighten it in place.
[image: image4.png]



[image: image5.png]



[image: image6.png]



四：将屏幕排线穿过定位板与PCB。连接到PCB背面的卡扣中。

Step 4: Pass the OLED screen cable through the switch plate and PCB. Connect it to the buckle located on the back of the PCB.
捏住黑色卡扣两端，轻轻拉开。将排线插入其中。然后轻轻折叠排线。（先插入卡扣再折叠排线比较容易操作。）
Pinch the ends of the black clip and gently pull to release it. Insert the cable into it. Then gently fold the cable. (It is easier to insert the buckle and then fold the cable.)
[image: image7.png]



注意事项：

1， 此PCB因功能较多，走线压力大，所以只支持3脚轴。（五角轴请自行剪脚）

2， 若为了追求RGB灯效，请选用轴的底壳，上盖都透光的轴体。

3， 具体使用说明请去群文件下载。

Precautions:

1. As there are many features in this PCB, the PCB comes with many traces. Thus, the PCB is limited to only plated mounted switches. You would have to trim off the addition two supporting legs if you are using a normal PCB mounted switch.

2. To pursue best lighting effects, please use RGB switches or those with transparent housing (i.e. Tealios etc.) 

3. For specific instructions, please download the user guide.
