How to upload the hex file onto your KMAC (key remap):

- Plug in another keyboard besides KMAC

- Unplug the USB cable from the KMAC

- Connect the KMAC’s USB cable again while pressing Caps Lock key (this will allow it to enter the binary upload mode and all the LEDs should be off)

- Start Flip program (latest version is 3.4.5 and you can download it here)

- Click on the IC icon (the first icon under File menu)

- It will say “Select a device”, choose “ATmega32U4” and then click OK

- After that, click the USB icon (it’ll say USB  Ctrl + U) and then click Open

- Next, click File menu and select “Load HEX file”

- Point it to the KMAC key map HEX file on your computer

- Click Run on the bottom left area (don’t change any options)

- After it finishes, click “Start Application” button on the bottom right

- Your KMAC LEDs should come back up

If you don’t have the correct driver installed, please follow the steps below:

- Right click on the Computer icon (on the desktop or start menu) and select Properties

- Choose Device Manager

- Locate the ATmega32U4FUD with exclamation mark in front (no driver installed)

- Click on it and then choose “Browse my computer for driver”

- Navigate to C:\Program Files (x86)\Atmel\Flip 3.4.3\usb (the correct driver should be in that location)

- Click Apply